RISC-V Codes

// =================== Design (DUT): Pipelined CPU ===================

`timescale 1ns/1ps         // Time unit = 1ns, precision = 1ps

`timescale 1ns/1ps         // (duplicate, harmless but redundant)

// ------------------- CPU Top Module -------------------

module pipelined_cpu #(

parameter ADDR_WIDTH = 8,  // Memory address width -> 256 locations(2^8)

parameter DATA_WIDTH = 32, // Instruction/data word size = 32-bit

parameter REG_COUNT  = 16  // Number of registers in register file

)(

input logic clk,            // Clock input

input logic reset           // Reset input (synchronous/asynchronous mix)

);

localparam MEM_DEPTH = (1<<ADDR_WIDTH); // Depth of instruction/data memories

// Memories and registers

logic [DATA_WIDTH-1:0] instr_mem [0:MEM_DEPTH-1]; // Instruction memory

logic [DATA_WIDTH-1:0] data_mem  [0:MEM_DEPTH-1]; // Data memory

logic [DATA_WIDTH-1:0] regfile   [0:REG_COUNT-1]; // Register file

// ---------------- Pipeline registers ----------------

// IF/ID latch

logic [DATA_WIDTH-1:0] if_id_instr, if_id_pc;

// ID/EX latch

logic [DATA_WIDTH-1:0] id_ex_pc;

logic [DATA_WIDTH-1:0] id_ex_rs1_val, id_ex_rs2_val, id_ex_imm;

logic [3:0] id_ex_rd, id_ex_rs1, id_ex_rs2;

logic [6:0] id_ex_opcode;

logic id_ex_is_load;

// EX/MEM latch

logic [DATA_WIDTH-1:0] ex_mem_alu_out, ex_mem_rs2_val;

logic [3:0] ex_mem_rd;

logic [6:0] ex_mem_opcode;

logic ex_mem_is_load;

// MEM/WB latch

logic [DATA_WIDTH-1:0] mem_wb_data;

logic [3:0] mem_wb_rd;

logic [6:0] mem_wb_opcode;

logic mem_wb_is_load;

// Program counter

logic [ADDR_WIDTH-1:0] pc;

// Branch signals

logic branch_taken_ex;                 // branch taken flag

logic [ADDR_WIDTH-1:0] branch_target_ex; // branch target address

// Control signals

logic stall;  // for hazards

logic flush;  // for branch flush

// Forwarding definitions

typedef enum logic [1:0] {F_NONE=2'b00, F_EX_MEM=2'b01, F_MEM_WB=2'b10} fwd_t;

fwd_t forwardA, forwardB;

//                   IF STAGE

always_ff @(posedge clk or posedge reset) begin

if(reset) begin

pc <= 0;                 // reset PC

if_id_instr <= 0;        // clear IF/ID instr

if_id_pc <= 0;           // clear IF/ID PC

flush <= 0;

end else begin

if(!stall) begin         // only fetch when no stall

if_id_instr <= instr_mem[pc]; // fetch instruction

if_id_pc <= pc;              // save PC in pipeline

if(branch_taken_ex) begin    // if branch taken

pc <= branch_target_ex;  // jump to target

flush <= 1;              // flush pipeline

end else begin

pc <= pc + 1;            // next sequential PC

flush <= 0;

end

end

end

end

//                   ID STAGE

always_ff @(posedge clk or posedge reset) begin

if(reset) begin

// Reset all ID/EX pipeline values

id_ex_pc <= 0;

id_ex_rs1_val <= 0;

id_ex_rs2_val <= 0;

id_ex_imm <= 0;

id_ex_rd <= 0;

id_ex_rs1 <= 0;

id_ex_rs2 <= 0;

id_ex_opcode <= 0;

id_ex_is_load <= 0;

end else begin

if(stall) begin

// Insert bubble (NOP) on stall

id_ex_pc <= 0;

id_ex_rs1_val <= 0;

id_ex_rs2_val <= 0;

id_ex_imm <= 0;

id_ex_rd <= 0;

id_ex_rs1 <= 0;

id_ex_rs2 <= 0;

id_ex_opcode <= 0;

id_ex_is_load <= 0;

end else begin

// Decode instruction

logic [3:0] rs1, rs2, rd;

logic [6:0] opcode;

logic [31:0] imm;

rs1 = if_id_instr[23:20];     // extract rs1 field

rs2 = if_id_instr[19:16];     // extract rs2 field

rd  = if_id_instr[27:24];     // extract rd field

opcode = if_id_instr[6:0];    // opcode

imm = {{16{if_id_instr[15]}}, if_id_instr[15:0]}; // sign-extended imm

id_ex_pc <= if_id_pc;          // pass PC

id_ex_rs1_val <= (rs1==0)?0:regfile[rs1]; // read rs1 value

id_ex_rs2_val <= (rs2==0)?0:regfile[rs2]; // read rs2 value

id_ex_imm <= imm;              // immediate

id_ex_rd <= rd;                // destination reg

id_ex_rs1 <= rs1;

id_ex_rs2 <= rs2;

id_ex_opcode <= opcode;        // opcode

id_ex_is_load <= (opcode==7'b0000100); // detect LOAD

end

end

end

//          Hazard Detection (Stall logic)

always_comb begin

stall = 0;

// If ID stage depends on result of a LOAD in EX -> stall

if(id_ex_is_load && (id_ex_rd!=0) && ((id_ex_rd==if_id_instr[23:20])||(id_ex_rd==if_id_instr[19:16])))

stall = 1;

end

//          Forwarding Unit (Data Hazards)

always_comb begin

forwardA = F_NONE;

forwardB = F_NONE;

// Forward from EX/MEM stage

if(ex_mem_rd!=0 && ex_mem_rd==id_ex_rs1 && ex_mem_opcode!=7'b0000101)

forwardA = F_EX_MEM;

else if(mem_wb_rd!=0 && mem_wb_rd==id_ex_rs1 && mem_wb_opcode!=7'b0000101)

forwardA = F_MEM_WB;

if(ex_mem_rd!=0 && ex_mem_rd==id_ex_rs2 && ex_mem_opcode!=7'b0000101)

forwardB = F_EX_MEM;

else if(mem_wb_rd!=0 && mem_wb_rd==id_ex_rs2 && mem_wb_opcode!=7'b0000101)

forwardB = F_MEM_WB;

end

//                   EX STAGE

logic [31:0] op_a, op_b, alu_result;

always_comb begin

// defaults

branch_taken_ex = 0;

branch_target_ex = 0;

// Select ALU operands considering forwarding

op_a = (forwardA==F_NONE)?id_ex_rs1_val:(forwardA==F_EX_MEM)?ex_mem_alu_out:mem_wb_data;

op_b = (forwardB==F_NONE)?id_ex_rs2_val:(forwardB==F_EX_MEM)?ex_mem_alu_out:mem_wb_data;

// ALU Operations

unique case(id_ex_opcode)

7'b0000001: alu_result = op_a + op_b;        // ADD

7'b0000010: alu_result = op_a - op_b;        // SUB

7'b0000011: alu_result = op_a + id_ex_imm;  // ADDI

7'b0000100: alu_result = op_a + id_ex_imm;  // LOAD addr

7'b0000101: alu_result = op_a + id_ex_imm;  // STORE addr

7'b0000110: alu_result = op_a & op_b;       // AND

7'b0000111: alu_result = op_a | op_b;       // OR

7'b0001001: alu_result = op_a ^ op_b;       // XOR

7'b0001010: alu_result = (op_a<op_b)?1:0;   // SLT

7'b0001111: alu_result = op_a * op_b;       // MUL

7'b0001000: begin                          // BEQ

if(op_a==op_b) begin

branch_taken_ex = 1;                // branch taken

branch_target_ex = id_ex_pc + id_ex_imm[ADDR_WIDTH-1:0]; // calc target

end

alu_result = 0;

end

default: alu_result = 0;

endcase

end

// EX/MEM pipeline register update

always_ff @(posedge clk or posedge reset) begin

if(reset) begin

ex_mem_alu_out <= 0;

ex_mem_rs2_val <= 0;

ex_mem_rd <= 0;

ex_mem_opcode <= 0;

ex_mem_is_load <= 0;

end else begin

ex_mem_alu_out <= alu_result;

ex_mem_rs2_val <= op_b;

ex_mem_rd <= id_ex_rd;

ex_mem_opcode <= id_ex_opcode;

ex_mem_is_load <= id_ex_is_load;

end

end

//                   MEM STAGE

always_ff @(posedge clk or posedge reset) begin

if(reset) begin

mem_wb_data <= 0;

mem_wb_rd <= 0;

mem_wb_opcode <= 0;

mem_wb_is_load <= 0;

end else begin

unique case(ex_mem_opcode)

7'b0000100: mem_wb_data <= data_mem[ex_mem_alu_out[ADDR_WIDTH-1:0]]; // LOAD

7'b0000101: begin

data_mem[ex_mem_alu_out[ADDR_WIDTH-1:0]] <= ex_mem_rs2_val; // STORE

mem_wb_data <= 0;

end

default: mem_wb_data <= ex_mem_alu_out; // ALU result forward

endcase

mem_wb_rd <= ex_mem_rd;

mem_wb_opcode <= ex_mem_opcode;

mem_wb_is_load <= ex_mem_is_load;

end

end

//                   WB STAGE

always_ff @(posedge clk or posedge reset) begin

if(!reset) begin

// Write back if not STORE and rd != 0

if(mem_wb_opcode!=7'b0000101 && mem_wb_rd!=0)

regfile[mem_wb_rd] <= mem_wb_data;

end

end

endmodule

// Code your testbench here

// or browse Examples

module testbench;

logic clk;

logic reset;

pipelined_cpu dut(.clk(clk), .reset(reset)); // instantiate DUT

// Clock generation

initial clk = 0;

always #5 clk = ~clk; // 10ns clock

// Function to pack instruction fields into 32-bit word

function automatic logic [31:0] pack(

input logic [3:0] rd, rs1, rs2,

input logic [15:0] imm,

input logic [6:0] op

);

logic [31:0] tmp = 0;

tmp[27:24] = rd;

tmp[23:20] = rs1;

tmp[19:16] = rs2;

tmp[15:0]  = imm;

tmp[6:0]   = op;

return tmp;

endfunction

// Task to initialize instruction and data memories

task init_memories();

for(int i=0;i<256;i++) begin

dut.instr_mem[i] = 0;

dut.data_mem[i] = 0;

end

for(int i=0;i<16;i++) dut.regfile[i] = 0;

// Program: ADDI, ADD, STORE, LOAD, ADD

dut.instr_mem[0] = pack(4'd1, 4'd0, 4'd0, 16'd5, 7'b0000011);  // x1=5

dut.instr_mem[1] = pack(4'd2, 4'd0, 4'd0, 16'd10,7'b0000011);  // x2=10

dut.instr_mem[2] = pack(4'd3, 4'd1, 4'd2, 16'd0, 7'b0000001); // x3=x1+x2

dut.instr_mem[3] = pack(4'd0, 4'd0, 4'd3, 16'd0, 7'b0000101); // STORE x3

dut.instr_mem[4] = pack(4'd4, 4'd0, 4'd0, 16'd0, 7'b0000100); // LOAD x4

dut.instr_mem[5] = pack(4'd5, 4'd4, 4'd1, 16'd0, 7'b0000001); // x5=x4+x1

// Data memory init

dut.data_mem[0] = 32'hDEADBEEF;

endtask

initial begin

$dumpfile("pipelined_cpu_waveform.vcd"); // dump waveforms

$dumpvars(0, testbench);

reset = 1; #20;    // Apply reset

reset = 0;

init_memories();   // initialize memories

#500;              // wait for simulation

// Display register file snapshot

$display("REGFILE snapshot:");

for(int i=0;i<8;i++) $display("r%0d = %h", i, dut.regfile[i]);

$display("MEM[0] = %h", dut.data_mem[0]);

$finish;

end

endmodule

// Simple interface to connect TB and verification components (placeholder for modularity)

interface tb_if(input bit clk, input bit rst_n);

logic valid;

logic [31:0] addr;

logic [31:0] data;

modport master (input clk, rst_n, valid, addr, data);

modport slave  (output clk, rst_n, valid, addr, data);

endinterface

// transaction.sv

class trans;

rand bit [31:0] addr;

rand bit [31:0] data;

rand bit        is_write;

function new(); endfunction

function string convert2string(); return $sformatf("addr=%0h data=%0h wr=%0b", addr, data, is_write); endfunction

endclass

// generator.sv

class gen;

rand trans t;

function new(); endfunction

task produce(output trans out);

out = new();

out.randomize();

endtask

endclass

// driver.sv

class drv;

virtual tb_if vif;

function new(virtual tb_if vif_ = null);

vif = vif_;

endfunction

task drive(trans t);

// This is a placeholder. In a real environment, driver would write to DUT via virtual interface.

$display("DRIVER driving: %s", t.convert2string());

endtask

endclass

// monitor.sv

class mon;

virtual tb_if vif;

function new(virtual tb_if vif_ = null);

vif = vif_;

endfunction

task observe();

$display("[MONITOR] time=%0t observing DUT state", $time);

endtask

task capture_trans(input trans t);

$display("[MONITOR] Captured trans: %s at time %0t", t.convert2string(), $time);

endtask

endclass

// scoreboard.sv

class sb;

bit [31:0] expected_mem [0:127];

function new(); endfunction

task check(input pipelined_cpu dut_ref);

// lightweight check: memory[0]

if (expected_mem[0] !== dut_ref.data_mem[0])

$display("[SCOREBOARD] mismatch mem[0] exp=%h got=%h", expected_mem[0], dut_ref.data_mem[0]);

else

$display("[SCOREBOARD] mem[0] OK %h", dut_ref.data_mem[0]);

endtask

endclass

// environment.sv

class env;

gen generator;

drv driver;

mon monitor;

sb scoreboard;

function new();

generator = new();

driver = new();

monitor = new();

scoreboard = new();

endfunction

task run(pipelined_cpu dut_ref);

trans t;

generator.produce(t);

driver.drive(t);

monitor.observe();

// prepare expected memory (simple example)

scoreboard.expected_mem[0] = dut_ref.regfile[3];

scoreboard.check(dut_ref);

endtask

endclass

// test.sv wrapper

module testbench_wrap;

initial begin

#300;

env e = new();

// pass DUT handle via $root (simplified). In real UVM you'd use virtual interfaces.

e.run($root.testbench.dut);

#50;

$display("TEST complete");

end

endmodule